APPENDIX A

Validation Methodology

| Field | Value |
| --- | --- |
| Document | Appendix A — Validation Methodology |
| Parent thesis | BIMGUARD AI — MAICEN M10 Final Master Project |
| Author | Mark Shane Haines |
| System under validation | BIMGUARD AI: Blue Halo seismic-bracing clearance engine and the five-mechanism corrosion rule stack (GC-001, CC-001, MC-001, MM-001, XM-001) |
| Validation dataset | 38 IFC models from 8 independent public repositories |
| Status | Methodology and presentation framework fixed in advance of the batch run; results tables in Section A.3 are unpopulated templates |
| Date | 23 August 2026 |

Purpose of this appendix This appendix fixes the validation protocol before any results exist. It states which models are tested, how they are processed, which numbers are recorded, how those numbers will be presented, and — critically — what ranges would count as a pass, a null result, or a failure. Declaring the interpretation criteria ahead of the run is what separates validation from post-hoc rationalisation of whatever the pipeline happened to produce. Sections A.1 and A.2 are complete and final. Section A.3 supplies empty tables and chart specifications ready to receive batch-run output. Section A.4 states the expected outcome ranges, derived from the synthetic and benchmark baselines already measured on this codebase.

Contents

A.1  Validation Dataset Overview	3

A.1.1  Scope and rationale	3

A.1.2  Selection criteria	3

A.1.3  Distribution	3

A.1.4  Source repositories	4

A.1.5  Material data quality classification	4

A.1.6  Model inventory	5

A.1.7  Sources investigated and excluded	6

A.1.8  Declared limitation — the industrial category	7

A.2  Testing Methodology	8

A.2.1  Automated batch processing pipeline	8

A.2.2  Harness provenance	8

A.2.3  Metrics collected per model	9

A.2.4  Corrosion engine scoring	9

A.2.5  Standards compliance basis	11

A.2.6  Clash severity classification	12

A.2.7  BCF export validity	12

A.2.8  Reproducibility and determinism	12

A.3  Results Presentation Framework	14

A.3.1  Table templates	14

A.3.2  Chart specifications	15

A.3.3  Summary statistics format	16

A.4  Interpretation Guide	18

A.4.1  What the results mean for Blue Halo algorithm validation	18

A.4.2  What the results mean for corrosion engine effectiveness	18

A.4.3  Expected outcome ranges	19

A.4.4  Threats to validity	20

A.5  Standards and Sources Cited in This Appendix	22

A.5.1  Standards	22

A.5.2  Dataset repositories	22

A.5.3  Project artefacts referenced	22

# A.1  Validation Dataset Overview

## A.1.1  Scope and rationale

BIMGUARD AI has, to this point, been exercised against synthetic fixtures: hand-built dataclasses in validate_blue_halo.py and a single purpose-generated IFC file (data/test_hospital_mep_scenario.ifc). Synthetic fixtures prove that the algorithm computes what it was written to compute. They cannot show that it survives contact with models authored by other people, in other tools, under other national conventions, at production element counts. That is the gap this dataset closes.

Thirty-eight IFC models were sourced from eight independently operated public repositories. Every download link was individually verified by HTTP request during compilation (HTTP 200 plus a real Content-Length), so the dataset is a set of working file downloads rather than a bibliography of landing pages. No model requires a login, a registration, or a payment, and every model carries a redistributable open licence (CC BY 3.0/4.0, MIT, or public-sector open data). A third party can therefore reconstruct the exact test corpus from this appendix alone, which is the reproducibility bar the validation is being held to.

## A.1.2  Selection criteria

Candidate models were admitted only if they satisfied every criterion below. The criteria were fixed before searching, and the exclusions in A.1.7 record where otherwise attractive sources failed them.

Table A.1.2 — Dataset selection criteria

| # | Criterion | Operational test | Why it matters |
| --- | --- | --- | --- |
| 1 | MEP-rich where possible | File contains IfcPipeSegment / IfcDuctSegment / IfcFlowSegment entities, ideally with material text | The corrosion engines score pipe material and media; a model with no pipes exercises nothing |
| 2 | Verified direct download | HTTP HEAD/GET returns 200 with a non-zero Content-Length pointing at the .ifc or .zip itself | Removes the single most common reproducibility failure in BIM research: dead sample-model links |
| 3 | No login, no payment | File retrievable by anonymous request | An examiner or replicator must be able to obtain the corpus without an account |
| 4 | Redistributable licence | CC BY 3.0/4.0, MIT, or public-sector open data on the hosting repository | Permits citation, quotation of entity counts, and re-hosting of derived results |
| 5 | Individually ≤ 100 MB | Measured download size, not vendor estimate | Keeps the batch run tractable on commodity hardware and inside repository limits |
| 6 | Building-scale, not component-scale | Model contains a spatial hierarchy (site/building/storey), not a single product | Manufacturer BIM libraries return switchgear and cooling units; those do not test spatial reasoning |
| 7 | Diverse building types | Classified as hospital / office / industrial-lab at compilation | Blue Halo clearance rules and importance factors are occupancy-dependent |
| 8 | Schema diversity | IFC2x3 and IFC4 both represented, with same-building twins where available | Parser robustness across schema versions is a distinct failure mode from geometry handling |

## A.1.3  Distribution

Table A.1.3 — Dataset distribution by building type

| Category | Models | Share | Row range | Character of the sample |
| --- | --- | --- | --- | --- |
| Hospital / Healthcare | 16 | 42% | 1–16 | Two full multi-discipline building sets (Medical-Dental Clinic; West Riverside Hospital) plus regional additions |
| Office / Commercial | 16 | 42% | 18–33 | Duplex benchmark set, RWTH DigitalHub, WBDG office prototype, KIT/IFC Wiki reference models, national-standard examples |
| Industrial / Laboratory | 6 | 16% | 34–39 | One genuine as-built laboratory plus flagged general-purpose and structural-steel robustness testers |
| Total | 38 | 100% | — | 8 independent repositories; 10 countries of origin; IFC2x3 and IFC4 both represented |

Row numbering follows the compilation working list, which skips 17 and 40; every row listed is a distinct file. Percentages are rounded to the nearest whole number.

## A.1.4  Source repositories

Table A.1.4 — Source repositories and licensing

| Repository | Models | Licence | Country / operator |
| --- | --- | --- | --- |
| buildingSMART Community-Sample-Test-Files (GitHub) | 11 | CC BY 4.0 | International / buildingSMART community |
| Hugging Face ifc-bench (mirrors OpenIFC Model Repository, Univ. of Auckland, and WBDG/NIBS) | 12 | CC BY 3.0 | NZ / US, mirrored |
| RWTH-E3D DigitalHub (GitHub) | 4 | MIT | Germany / RWTH Aachen |
| KIT IFC Examples via IFC Wiki | 2 | Free with attribution | Germany / KIT |
| TIB Hannover — DuraArk BuildingData | 4 | Open data | Germany / TIB |
| Zenodo (record 7948116) | 1 | Open access CC | Portugal / FEUP |
| Generalitat Valenciana — Conselleria d'Habitatge | 2 | Public-sector open data | Spain |
| STEP Tools sample IFC data (NIST/AISC conversion) | 2 | Public / NIST | US |

Counts sum to 38, matching the inventory in Table A.1.6. Zip archives bundling several discipline files are counted once, as one inventory row. The ifc-bench figure spans two upstream collections that Hugging Face mirrors — the Auckland OpenIFC repository (rows 6–14) and the WBDG/NIBS Common BIM Files (rows 28–30) — which is why it is the largest single entry.

## A.1.5  Material data quality classification

Every model carries a material-data tier assigned at compilation. The tier determines which engines can meaningfully run against it, and therefore which denominator each result is reported over. Reporting a corrosion-engine finding rate over all 38 models when only a subset carries material text would understate the engines by construction.

Table A.1.5 — Material data quality tiers

| Tier | Definition | Engines exercised |
| --- | --- | --- |
| Full | Pipe/duct entities carry material, size and system-type properties sufficient for corrosion and clearance scoring | Blue Halo + all five corrosion engines |
| Partial | Geometry and system type present, material properties sparse or schema-dependent | Blue Halo; corrosion engines expected to emit data-quality issues rather than risk bands |
| Architecture-only | No MEP systems in the file | Blue Halo clash geometry and parser robustness only |
| n/a (discipline) | Structural, electrical or fire-alarm file with no piped systems | Used as clash candidates for Blue Halo halos generated from paired MEP files |

## A.1.6  Model inventory

The full corpus is listed below by category. Sizes are measured download sizes. "n.v." denotes a property not verified beyond the HTTP check; "content-verified" denotes a file downloaded and inspected during compilation.

Table A.1.6a — Hospital / healthcare models (16)

| # | Model file | Systems | MB | Schema | Material tier |
| --- | --- | --- | --- | --- | --- |
| 1 | Clinic_Architectural.ifc | Walls, doors, windows, rooms | 12.40 | IFC2x3 | Architecture-only |
| 2 | Clinic_Structural.ifc | Beams, columns, slabs | 18.18 | IFC2x3 | n/a (structural) |
| 3 | Clinic_HVAC.ifc | Ductwork, AHU, terminals | 25.67 | IFC2x3 | Full |
| 4 | Clinic_Electrical.ifc | Cable trays, fixtures, panels | 6.49 | IFC2x3 | n/a (electrical) |
| 5 | Clinic_Plumbing.ifc | 378 pipe segments/fittings; Copper, PVC, CPVC, Steel, Brass | 53.25 | IFC2x3 | Full — content-verified |
| 6 | west_riverside_hospital — arc_ifc4 | Architecture, 15,316 elements | 77.19 | IFC4 | Architecture-only |
| 7 | west_riverside_hospital — str_ifc4 | 2,915 elements; 1,970 beams, 255 columns | 6.18 | IFC4 | n/a (structural) |
| 8 | west_riverside_hospital — mech_ifc4 | 59,215 elements; 3,916 pipe segments, 1,064 terminals | 69.66 | IFC4 | Full |
| 9 | west_riverside_hospital — plumb_ifc4 | 26,942 elements; 4,308 pipe segments | 22.66 | IFC4 | Full |
| 10 | west_riverside_hospital — elec_ifc4 | 6,305 elements; 1,272 light fixtures | 4.23 | IFC4 | n/a (electrical) |
| 11 | west_riverside_hospital — fire_ifc4 | 874 fire-alarm devices | 0.86 | IFC4 | n/a |
| 12 | west_riverside_hospital — sprinkle_ifc4 | 38,255 elements; 6,228 pipe segments, 1,354 heads | 32.42 | IFC4 | Full |
| 13 | west_riverside_hospital — mech_ifc2x3 | Schema twin of row 8 | 75.12 | IFC2x3 | Full |
| 14 | west_riverside_hospital — plumb_ifc2x3 | Schema twin of row 9 | 23.84 | IFC2x3 | Full |
| 15 | NBU_MedicalClinic (zip: ARC+MEP+HVAC+ELE+STR) | Full discipline set | 78.90 | IFC2x3 | n.v. |
| 16 | GVA Sanitario.ifc | Architecture only | 6.64 | n.v. | Architecture-only |

Row 15 caveat: the MEP sub-file inside NBU_MedicalClinic_ifc.zip is approximately 207 MB uncompressed and breaches the 100 MB per-file cap once extracted; the other four discipline files do not. Row 5 is the strongest single file in the corpus for the galvanic and cross-material engines.

Table A.1.6b — Office / commercial models (16)

| # | Model file | Systems | MB | Schema | Material tier |
| --- | --- | --- | --- | --- | --- |
| 18 | Duplex_A_20110907.ifc | Architecture | 2.27 | IFC2x3 | Architecture-only |
| 19 | Duplex_Electrical_20121207.ifc | Electrical | 1.53 | IFC2x3 | n/a |
| 20 | Duplex_MEP_20110907.ifc | Combined MEP | 17.04 | IFC2x3 | Full |
| 21 | Duplex_Plumbing_20121113.ifc | Pipes | 30.09 | IFC2x3 | Full |
| 22 | AC20-Institute-Var-2.ifc | Architecture plus some MEP | 10.43 | IFC4 | Partial |
| 23 | AC20-FZK-Haus.ifc | Architecture | 2.45 | IFC4 | Architecture-only |
| 24 | DigitalHub_FM-ARC_v2.ifc | Architecture | 8.60 | IFC4 (Ref. View) | Architecture-only |
| 25 | DigitalHub_FM-HZG_v2.ifc | Heating pipework | 19.92 | IFC4 | Full |
| 26 | DigitalHub_FM-LFT_v2.ifc | Ventilation ductwork | 12.15 | IFC4 | Full |
| 27 | DigitalHub_FM-SAN_v2.ifc | Plumbing / sanitary | 24.01 | IFC4 | Full |
| 28 | wbdg_office — arc.ifc | Architecture | 3.91 | n.v. | Architecture-only |
| 29 | wbdg_office — mep.ifc | 642 duct segments, 1,569.99 m ductwork (published ground truth) | 39.95 | n.v. | Full |
| 30 | wbdg_office — str.ifc | Beams, columns | 10.55 | n.v. | n/a |
| 31 | Molio_with_URIs.ifc | Architecture with URI classification | 70.52 | IFC2x3 | Architecture-only |
| 32 | GVA Administrativo.ifc | Architecture only | 6.88 | n.v. | Architecture-only |
| 33 | IFC Schependomlaan.ifc | Architecture (design model) | 47.00 | IFC2x3 | Architecture-only |

Row 29 carries published duct-length ground truth in its dataset QA pairs (Supply Air 860.04 m, Return Air 697.93 m, Exhaust Air 12.02 m) and is therefore the corpus's only external check on halo volume arithmetic. Rows 18–21 and 33 are among the most heavily cited open BIM benchmarks in the construction-technology literature, permitting cross-reference of clash counts against published studies. Row 33 is mixed residential rather than pure office and is flagged as such.

Table A.1.6c — Industrial / laboratory models (6)

| # | Model file | Systems | MB | Schema | Material tier |
| --- | --- | --- | --- | --- | --- |
| 34 | craslabbim.ifc | As-built research laboratory, full building (CRAS Lab, FEUP) | 64.42 | n.v. | Partial |
| 35 | SGD_BODO_ifc.zip (ARC+PLB+VENT) | Plumbing and ventilation | 31.11 (zip) | n.v. | n.v. |
| 36 | Academic_Autodesk_ifc.zip (Basic + Advanced) | Architecture, mixed | 7.03 (zip) | n.v. | n.v. |
| 37 | Autodesk_210-King_ifc.zip | Structural and architecture, steel frame | 25.49 (zip) | n.v. | n.v. |
| 38 | aisc_sculpture_brep.ifc | Structural steel, brep solid geometry | 0.53 | IFC2x3 | n/a (steel) |
| 39 | aisc_sculpture_param.ifc | Same steel structure, parametric swept solids | 0.30 | IFC2x3 | n/a (steel) |

Rows 38 and 39 are the same steel assembly in two geometric representations and exist in the corpus specifically to test the geometry-handling path independently of MEP content. Row 35's building programme is not confirmed as strictly industrial and its extracted discipline files may individually approach the 100 MB cap.

## A.1.7  Sources investigated and excluded

Recording what was rejected, and why, is part of the method. Each entry below was a plausible candidate that failed a stated criterion.

Table A.1.7 — Excluded sources

| Source | Reason for exclusion | Criterion failed |
| --- | --- | --- |
| University of Auckland Open IFC Model Repository | All /Model/Details/ and /_models/ paths return an empty page; the canonical host is broken. West Riverside Hospital recovered only via the Hugging Face ifc-bench mirror | 2 — verified download |
| Nordic Sustainable Construction BIM4LCA | ARC.zip is 1.5 GB and HVAC.zip 870 MB, bundling multiple variants and native non-IFC formats; no individual file isolable without a multi-gigabyte download | 5 — ≤100 MB |
| TUM GNI BIM Dataset (Zenodo 19722012) | Legitimate CC BY 4.0 source but delivered only as 503 MB and 221 MB bulk archives with no per-model links | 5 — ≤100 MB |
| Holter_Tower_10.ifc (DuraArk/BIMData catalogue) | Listed Dropbox link returns an HTML page, not the file | 2 — verified download |
| NBU_OfficeBuilding | The catalogue's source link is a byte-for-byte duplicate of the NBU_MedicalClinic zip URL; the office entry is mislabelled at source | Data integrity |
| Kaggle IFC datasets ("5000 IFC/RVT Projects", "IFC Sample Models") | Require account login | 3 — no login |
| Manufacturer BIM libraries (Siemens, Stulz, GEA) | Single-component product files — a switchgear, a cooling unit, a cleanroom door — not building models | 6 — building-scale |

## A.1.8  Declared limitation — the industrial category

The industrial/laboratory category is the corpus's weak point and is presented as such rather than padded. Genuinely public, building-scale IFC models of factories, warehouses, data centres, cleanrooms or power plants with rich MEP content are scarce on the open web. After searching buildingSMART, GitHub, Zenodo, Figshare, university repositories (MIT, Stanford, Delft, ETH Zurich, RWTH Aachen, TUM, KIT) and the DuraArk collections, craslabbim.ifc (row 34) was the only confirmed genuine industrial or laboratory building with strong provenance. Rows 35–39 are included transparently as general-purpose and structural-steel robustness testers, flagged in the inventory rather than dressed up as industrial buildings.

The consequence for interpretation is stated in A.4.4: conclusions about industrial-facility behaviour rest on n = 1 and must be reported as indicative, not established.

# A.2  Testing Methodology

## A.2.1  Automated batch processing pipeline

Each model is processed by a single non-interactive pass. The pipeline is staged so that cost and failure can be attributed to a specific stage rather than to "the run"; a model that fails at parse yields a recorded parse failure and no downstream rows, rather than silently contributing zeros to the aggregate.

Table A.2.1 — Batch pipeline stages

| Stage | Operation | Implementation | Recorded on failure |
| --- | --- | --- | --- |
| 0 — Acquire | Fetch the model from its verified URL; record HTTP status, byte count and SHA-256 | Batch driver | URL dead / size mismatch |
| 1 — Parse | ifcopenshell.open(); resolve schema version and project unit scale to millimetres | unit_scale_to_mm() in halo_volume_generator | Parse error, schema unsupported |
| 2 — Extract | Enumerate MEP and structural elements; triangulate to world-coordinate meshes via ifcopenshell.geom.iterator | performance_benchmark.py ingest stage | Geometry resolution failure per element |
| 3 — Halo | Generate one clearance envelope per in-scope element from the loaded jurisdiction config | generate_halo_volume() / generate_halo_volume_from_geometry() | Element skipped, no placement or usable shape |
| 4 — Clash | Broad-phase uniform spatial hash, then exact mid-phase AABB overlap between halos and candidate geometry | detect_halo_clash() / detect_halo_clash_against_geometry() | — |
| 5 — Corrode | Coerce each piping element into the five engine input models and score it | compliance_runner.py coercion adapters plus the XM-001/MM-001 compare() functions | Data-quality issue emitted, never a default score |
| 6 — Aggregate | Normalise every finding to the common issue schema {band, score, details} | issue_adapter.py / issue_schema.py | Schema violation |
| 7 — Export | Emit a BCF 2.1 ZIP of all findings and a Pset_HaloReservation JSON per halo | blue_halo_bcf_exporter.py, bcf_generator.py | Invalid archive structure |
| 8 — Record | Write the per-model metric row defined in Table A.2.3 | Batch driver | — |

## A.2.2  Harness provenance

The batch driver composes existing, already-exercised harnesses rather than reimplementing the pipeline. This matters for validity: the code path under test in the batch run is the same code path the earlier synthetic validation passed, differing only in its input.

Table A.2.2 — Existing harnesses composed by the batch run

| Harness | What it already covers | Role in the batch run |
| --- | --- | --- |
| validate_blue_halo.py | Core generation and detection against hand-built geometry, real EN 1998-1 + DIN 4149 config; 11/11 checks passing | Supplies the pass criteria replicated per model |
| test_real_ifc_pipeline.py | Full ifcopenshell path on one fixture: placement matrices, tessellated vertices, metre-to-millimetre scaling, BCF and Pset export | Template for the per-model end-to-end pass |
| performance_benchmark.py | Stage-separated timing and memory at 100–20,000 elements, n = 7 repeats, median and IQR reporting | Supplies the throughput baselines in Table A.4.3 |
| compliance_runner.py | Coercion of generic elements into GCElement / CCElement / MICElement and normalisation to {band, score, details} | Corrosion scoring stage |
| scripts/verify_anode_convention.py | Verification of the galvanic series sign convention against the seeded GC-001 payload | Pre-run integrity gate on the shared galvanic series |

## A.2.3  Metrics collected per model

One row is written per model. Every field is either measured or explicitly null; no field is imputed.

Table A.2.3 — Per-model metric schema

| Field | Type | Definition |
| --- | --- | --- |
| model_id | int | Inventory row number from Table A.1.6 |
| sha256 | string | Hash of the downloaded file, pinning the artefact the row describes |
| schema_version | string | IFC2x3 or IFC4, read from the file header |
| parse_seconds | float | Wall-clock duration of ifcopenshell.open() |
| element_count_total | int | All IfcProduct instances with geometry |
| element_count_mep | int | IfcPipeSegment, IfcDuctSegment, IfcFlowSegment and fittings |
| triangulate_seconds | float | Duration of the geometry iterator over in-scope elements |
| halos_generated | int | Clearance envelopes successfully produced |
| halos_skipped | int | Elements with no resolvable placement or shape representation |
| halo_seconds | float | Duration of the halo generation stage alone |
| halos_per_second | float | halos_generated / halo_seconds |
| mean_halo_volume_m3 | float | Arithmetic mean of the generated envelope volumes |
| candidate_pairs | int | Pairs surviving the broad-phase spatial hash |
| clashes_total | int | Pairs confirmed by exact AABB overlap |
| clashes_critical / major / minor | int | Clash counts by the severity bands in Table A.2.6 |
| corrosion_findings_by_engine | map | Count of issues raised, keyed by GC-001, CC-001, MC-001, MM-001, XM-001 |
| corrosion_bands | map | Count of findings by risk band, per engine |
| data_quality_issues | int | Findings emitted because required properties were absent — a coverage measure, not a risk measure |
| materials_observed | list | Distinct pipe material strings encountered |
| unmapped_pairings | int | Material/media combinations absent from the MM-001 matrix |
| bcf_bytes | int | Size of the exported BCF 2.1 archive |
| bcf_valid | bool | Result of the structural check in Table A.2.7 |
| peak_rss_mb | float | Peak resident set size delta across the model's processing |
| failure_stage | string¦null | Pipeline stage at which processing stopped, if any |

## A.2.4  Corrosion engine scoring

Five rule engines score each piping element. They are independent mechanisms, not a single score decomposed: an element may be benign under four and critical under the fifth, and that is the intended behaviour. Each engine carries its own weighted composite and its own band cut-offs, taken from the ruleset it implements.

Table A.2.4a — The five corrosion engines

| Ruleset | Mechanism | Composite score | Bands | Principal standards |
| --- | --- | --- | --- | --- |
| GC-001 | Galvanic corrosion | 0.50 × voltage_risk + 0.30 × area_ratio_risk + 0.20 × environment_multiplier | Low < 0.35; Medium 0.35–0.65; High 0.65–0.85; Critical > 0.85 | NASA-STD-6012; WorldStainless/Euro Inox; AUCSC; IMOA; Prosoco TN 104; AGA |
| CC-001 | Crevice corrosion | 0.35 × geometry_risk + 0.40 × CCT_adequacy + 0.25 × environment_severity | Low < 0.30; Medium 0.30–0.55; High 0.55–0.80; Critical > 0.80 | EN ISO 15329:2007; ASTM G48 Method B; CIRIA C692; CIBSE Guide G; EN 1993-1-4 |
| MC-001 | Microbially influenced corrosion | 0.35 × flow_velocity_risk + 0.30 × temperature_risk + 0.25 × dead_leg_risk + 0.10 × material_susceptibility | Low < 0.25; Medium 0.25–0.50; High 0.50–0.75; Critical > 0.75 | CIBSE TM13:2013; HSE HSG274 Pts 1–3; BS 8552:2012; ASTM G-187; EN ISO 9308-1; WHO GDWQ 4th ed. |
| MM-001 | Material–media compatibility | 0.40 × compatibility_score + 0.35 × environment_severity + 0.25 × temperature_stress | Compliant < 0.35; Medium 0.35–0.60; High 0.60–0.85; Critical ≥ 0.85 | ASTM B117; ASTM G48; CIBSE Guide G; EN 12952-12; IMOA; CDA |
| XM-001 | Cross-material contamination | Galvanic couple voltage resolved from the shared GC-001 series, suppressed below the environment threshold (default 0.25 V) | Medium < 0.50 V; High 0.50–0.85 V; Critical ≥ 0.85 V | BS 8539; NASA-STD-6012; EN ISO 15329:2007 (via MM-001 environment table); IMOA |

Two design decisions in the rule packs bear directly on how results must be read:

- Single-source tables. XM-001 deliberately contains no galvanic series and no environment table of its own; it is injected with the GC-001 series (loaded from the BIMGUARD-GC-001 catalogue) and the MM-001 environment severities at runtime. There is one series and one environment table in the system, so a divergence between engines cannot arise from table drift.

- No silent defaulting. A material/media pairing absent from the MM-001 matrix must emit a data-quality issue, never a default score. An unmapped pairing scored as 0.0 would be reported as compliant, which would be a false negative dressed as a pass. This is why data_quality_issues and unmapped_pairings are first-class metrics in Table A.2.3.

Table A.2.4b — Declared calibration limitations carried into the run

| Ruleset | Carried limitation |
| --- | --- |
| MM-001 | The rankings reflect corrosion-engineering consensus; the two-decimal scores and predicted lifespans are a calibration authored for this ruleset, not digits quoted from the cited standards. Each citation names the standard governing the mechanism, not a document the number was read from. |
| MM-001 | Water chemistry (chloride ppm, pH, hardness, dissolved oxygen) and velocity are absent from PipingElement. Copper Type 2 pitting in soft-water regions is understated; copper erosion-corrosion above ~1.5 m/s is not modelled. |
| MM-001 | T5_industrial environment severity (0.85) is provisional — the only environment value with no CC-001 wetting-class counterpart, interpolated rather than borrowed. |
| XM-001 | The galvanic series was verified as an anodic index (higher value = anode) against the GC-001 payload as last committed. If the Supabase-hosted asset was edited after seeding, the engines read a different table; re-verification without --from-seeder is required once credentials permit. |
| XM-001 | One internally inconsistent adjacent pair persists in the series: ss316_active (+0.22, noble = false) sits below copper (+0.28, noble = true), so for that couple the potential ordering and the noble flag disagree. XM-001 resolves direction from the flag and is unaffected, but the series needs a corrosion reviewer. |

These limitations are pre-declared so that engine output is interpreted as a ranking of relative risk under a stated calibration, not as an absolute engineering verdict. See A.4.2.

## A.2.5  Standards compliance basis

Blue Halo clearance geometry is driven by a jurisdiction configuration generated from a standards research summary. Four standards are represented; the batch run uses the combined European configuration as its primary case, with the US pair available as an alternate jurisdiction for the sensitivity comparison in Table A.3.1f.

Table A.2.5a — Source standards and extracted parameters

| Standard | Jurisdiction | Restraint spacing (transverse / longitudinal) | Pipe threshold | Min. clearance from structure | Brace angle | Importance factor | Citation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN 1998-1:2020 | EU | 1.0 m / 1.5 m | 63 mm dia. | 150 mm | 35–70° | 1.0 (1.5 critical facilities) | 5.3.2.3 |
| DIN 4149:2022 | Germany | 1.2 m / 1.8 m | 75 mm dia. | 200 mm | 40–65° | 1.0 (1.6 hospitals) | 8.2.4 |
| NFPA 13-2021 | US | 40 in / 60 in | 2.5 in dia. | 12 in | 30–60° | 1.0 (1.5 hospitals) | 18.3.2.1 |
| ASCE 7-22 | US | 48 in / 72 in | 8 in duct dia. | 18 in | 30–60° | 1.0 (1.5 hospitals) | 12.10.4.2 |

The combined EN 1998-1 + DIN 4149 configuration resolves each parameter by the conservative reading, and the resolution rule is recorded in the configuration itself rather than applied silently:

- Restraint spacing: per-axis minimum of the two standards — tighter spacing governs, because it requires more restraints.

- Clearance from structure: maximum of the two (200 mm) — the larger clearance produces an envelope satisfying both.

- Pipe diameter threshold: minimum of the two (63 mm) — the lower threshold brings more pipes into scope.

- Brace angle: intersection of both ranges (40–65°) — the band any brace satisfying both standards must fall in.

- Importance factors: per-key maximum — the higher factor demands the greater restraint capacity.

Table A.2.5b — Configuration data gaps declared before the run

| Gap | Effect on results |
| --- | --- |
| No duct-area bracing threshold in either standard | Duct in-scope determination cannot be made from the standard; duct halos are generated unconditionally and reported separately |
| seismic_zone_addition_mm and hospital_addition_mm absent from both standards; left at 0 mm rather than guessed | Hospital and standard-occupancy clearances are numerically identical (200 mm). A hospital-versus-standard comparison will show no difference, and that null is a data gap, not an algorithm defect |
| adjacent_system_mm absent from both standards; left at 0 mm | System-to-system clearance additions are not modelled |
| hazard_factor_default absent from both standards; left null | Site hazard scaling is not applied |
| No per-brace-hardware breakdown in either standard | All four brace variants (angle_fire, angle_mechanical, cable, rod) share one clearance value and therefore produce identical envelope volumes; brace-type comparison will be degenerate |
| brace_types[*].base_footprint_mm are generic hardware placeholders; standard_sizes empty | Footprint-driven results are indicative pending a hardware catalogue cross-reference |
| Citations are section references, not full bibliographic records | Traceability is to clause level; full citations remain outstanding |

Thirteen gaps are recorded in the configuration's own metadata.data_gaps field; the seven above are those that change how a result should be read. Declaring them here prevents a degenerate result being reported later as a finding.

## A.2.6  Clash severity classification

Severity is assigned from the overlap volume as a fraction of the halo volume. The thresholds are a generic spatial heuristic, not a banding taken from any standard, and are stated here so that any distribution reported in A.3 can be re-derived or re-banded by a reader who disagrees with them.

Table A.2.6 — Severity bands and BCF handling

| Severity | Overlap ratio | BCF priority | Due date offset |
| --- | --- | --- | --- |
| Critical | ≥ 25% of halo volume | Critical | 2 days |
| Major | ≥ 5% and < 25% | Major | 7 days |
| Minor | < 5% | Minor | 21 days |

## A.2.7  BCF export validity

A BCF archive is counted valid only if it satisfies every structural condition below. Byte size alone is not evidence of a usable export.

Table A.2.7 — BCF 2.1 validity conditions

| # | Condition |
| --- | --- |
| 1 | Archive contains a bcf.version entry declaring BCF 2.1 |
| 2 | Archive contains a project.bcfp entry |
| 3 | One GUID-named topic folder exists per reported finding — folder count equals finding count exactly |
| 4 | Each topic folder contains markup.bcf, viewpoint.bcfv and snapshot.png |
| 5 | Every markup.bcf carries a populated DueDate, derived from severity per Table A.2.6 |
| 6 | Every topic references a component GUID that exists in the source model |
| 7 | The archive opens without error in a standards-compliant BCF reader |

Alongside the BCF archive, each halo is exported as a Pset_HaloReservation property set (SchemaVersion 0.1.0) carrying the source GlobalId, IFC class, brace type and rule variant, clearance in millimetres, envelope volume, and both element and halo bounding boxes. The property set must be JSON-serialisable and round-trip without loss.

## A.2.8  Reproducibility and determinism

- Determinism. No randomness and no wall-clock-dependent value feeds any decision. Timestamps appear in output dataclasses but are never compared or asserted upon. Counts that should not vary between repeats — triangle counts, envelope volumes, interfering pairs — are checked for agreement across repeats rather than averaged.

- Repeats and statistics. Timing scenarios are run n = 7 and reported as median with inter-quartile range, not mean with standard deviation: a benchmark timing is bounded below by the true cost of the work and unbounded above by scheduler interference, so the distribution is right-skewed and a single stall drags the mean while leaving the median untouched.

- Instability flagging. Any metric whose IQR exceeds 20% of its median is flagged unstable in the output rather than quietly reported. A large relative spread on a sub-10 ms median is measurement noise; the same spread on a multi-second median is not, and no conclusion rests on it without further repeats.

- Environment capture. Python version, platform, processor, CPU count, total RAM, numpy version and ifcopenshell version are recorded with every run and reported with the results.

- Artefact pinning. Each model row records the SHA-256 of the file actually processed, so a later re-run against a silently updated upstream file is detectable rather than invisible.

# A.3  Results Presentation Framework

The tables in this section are the final reporting format, fixed before the data exists. They are reproduced here unpopulated; each cell marked "—" receives a measured value from the batch run. Fixing the format in advance removes the temptation to select a presentation that flatters whatever the run produces.

## A.3.1  Table templates

Table A.3.1a — Per-model processing summary (one row per model; 38 rows)

| # | Model | Elements | MEP elem. | Parse (s) | Halos | Halos/s | Clashes | BCF valid | Fail stage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Clinic_Architectural.ifc | — | — | — | — | — | — | — | — |
| … | … | … | … | … | … | … | … | … | … |
| 39 | aisc_sculpture_param.ifc | — | — | — | — | — | — | — | — |
|  | Median (all models) | — | — | — | — | — | — | — |  |
|  | IQR | — | — | — | — | — | — | — |  |

Table A.3.1b — Clash severity distribution by building type

| Building type | Models | Halos | Candidate pairs | Clashes | Critical | Major | Minor | Confirmation rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hospital | 16 | — | — | — | — | — | — | — |
| Office | 16 | — | — | — | — | — | — | — |
| Industrial / lab | 6 | — | — | — | — | — | — | — |
| All | 38 | — | — | — | — | — | — | — |

Confirmation rate = clashes ÷ candidate pairs, i.e. the proportion of broad-phase candidates surviving exact AABB testing. It measures spatial-index selectivity, not clash prevalence.

Table A.3.1c — Corrosion engine findings by pipe material

| Pipe material | Elements scored | GC-001 | CC-001 | MC-001 | MM-001 | XM-001 | Data-quality issues |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Copper (C12200) | — | — | — | — | — | — | — |
| Carbon steel | — | — | — | — | — | — | — |
| Galvanised steel | — | — | — | — | — | — | — |
| Stainless SS304 | — | — | — | — | — | — | — |
| Stainless SS316 | — | — | — | — | — | — | — |
| Brass | — | — | — | — | — | — | — |
| PVC | — | — | — | — | — | — | — |
| CPVC | — | — | — | — | — | — | — |
| Unrecognised / absent | — | — | — | — | — | — | — |
| Total | — | — | — | — | — | — | — |

Cell values are finding counts. A zero is a result: it means the engine ran and raised nothing. An empty denominator (elements scored = 0) is not a result and must be reported as no-coverage, not as compliance.

Table A.3.1d — Risk band distribution per engine

| Engine | Findings | Low / compliant | Medium | High | Critical | Median score | IQR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GC-001 galvanic | — | — | — | — | — | — | — |
| CC-001 crevice | — | — | — | — | — | — | — |
| MC-001 MIC | — | — | — | — | — | — | — |
| MM-001 material–media | — | — | — | — | — | — | — |
| XM-001 cross-material | — | — | — | — | — | — | — |

Table A.3.1e — BCF export validity (conditions per Table A.2.7)

| Building type | Archives produced | Topics | C1–C2 pass | C3 folder = finding count | C4 all 3 files present | C5 due date | Valid overall |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hospital | — | — | — | — | — | — | — |
| Office | — | — | — | — | — | — | — |
| Industrial / lab | — | — | — | — | — | — | — |
| All | — | — | — | — | — | — | — |

Table A.3.1f — Standards sensitivity (same models, alternate jurisdiction configuration)

| Configuration | Clearance (mm) | Angle band | Halos | Mean halo vol. (m³) | Clashes | Critical | Δ vs. baseline |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN 1998-1 + DIN 4149 (baseline) | 200 | 40–65° | — | — | — | — | — |
| NFPA 13 + ASCE 7-22 | — | 30–60° | — | — | — | — | — |
| EN 1998-1 alone | 150 | 35–70° | — | — | — | — | — |
| DIN 4149 alone | 200 | 40–65° | — | — | — | — | — |

Table A.3.1g — Parser robustness, IFC2x3 vs IFC4 schema twins

| Building / discipline | IFC2x3 elements | IFC4 elements | Δ elements | IFC2x3 halos | IFC4 halos | Δ halos | Volume agreement |
| --- | --- | --- | --- | --- | --- | --- | --- |
| West Riverside — mechanical (rows 13 / 8) | — | — | — | — | — | — | — |
| West Riverside — plumbing (rows 14 / 9) | — | — | — | — | — | — | — |
| AISC sculpture — brep / parametric (rows 38 / 39) | — | — | — | — | — | — | — |

Rows 13/8 and 14/9 are the same building exported to two schemas; rows 38/39 are the same steel assembly in two geometric representations. Element counts may legitimately differ between schemas; envelope volumes for corresponding elements should not.

## A.3.2  Chart specifications

Four figures accompany the tables. Each specification states the encoding and, more importantly, what the figure is supposed to let a reader see.

Figure A.1 — Clash severity distribution

| Property | Specification |
| --- | --- |
| Type | Horizontal stacked bar, one bar per building type, plus an "all models" bar |
| X axis | Proportion of clashes (0–100%), with absolute counts annotated at the bar end |
| Segments | Critical / Major / Minor, ordered severe-to-benign left to right |
| Colour | Sequential single-hue ramp, darkest for Critical; not red-amber-green, which implies a verdict the severity heuristic does not support |
| Reads as | Whether severity composition differs by building type. If hospital models show a heavier critical tail than office models, that is congestion, not a different algorithm |
| Caveat to print | Severity thresholds are a spatial heuristic (Table A.2.6), not a standards banding |

Figure A.2 — Material risk heatmap

| Property | Specification |
| --- | --- |
| Type | Matrix heatmap, materials on rows, the five engines on columns |
| Cell value | Median composite score for that material under that engine; count annotated in the cell |
| Colour | Sequential ramp on score, with a visually distinct hatched fill for "no coverage" (zero elements scored) |
| Critical detail | No-coverage cells must not render as low risk. A blank denominator and a benign score are opposite findings and must not share a colour |
| Reads as | Which mechanisms discriminate between materials and which return flat results across the corpus. A column that is uniform across every material is an engine that is not discriminating on this data |
| Caveat to print | Scores are relative rankings under the calibration declared in Table A.2.4b, not absolute engineering verdicts |

Figure A.3 — Throughput scaling

| Property | Specification |
| --- | --- |
| Type | Log-log scatter with fitted trend, one point per model |
| X axis | Element count |
| Y axis | Halo generation seconds (stage 3 only, excluding parse and triangulation) |
| Overlay | The synthetic scaling baseline from Table A.4.3 as a reference line |
| Reads as | Whether real-model generation cost stays linear in element count and lands on the synthetic baseline. Systematic elevation above the line indicates real geometry costs more than synthetic; curvature indicates a super-linear term |
| Caveat to print | Points are single runs per model, not n = 7 repeats; scatter includes host noise |

Figure A.4 — Halo volume distribution

| Property | Specification |
| --- | --- |
| Type | Box plot per building type, log-scaled y |
| Y axis | Envelope volume (m³) per halo |
| Reference | Synthetic baseline band (13.5–21.5 m³ mean) shaded behind the boxes |
| Reads as | Whether envelope sizes on real models are physically plausible. Volumes orders of magnitude off the baseline indicate a unit-scaling fault, which is the most likely silent failure in the IFC path |
| Caveat to print | Envelope volume is driven by element bounding-box size; long straight runs legitimately produce large envelopes |

## A.3.3  Summary statistics format

Every aggregate figure reported in the results chapter follows the conventions below, so that a number's meaning does not depend on where in the document it appears.

- Central tendency. Median with inter-quartile range, written as median (IQR). Means and standard deviations are not used for timing figures, for the right-skew reason stated in A.2.8.

- Counts. Reported as absolute count with its denominator, never as a bare percentage: "14 of 22 models (64%)", not "64%".

- Coverage. Every rate carries the population it is computed over. Corrosion findings are reported over elements scored, not over elements present, and the difference between the two is itself reported as coverage.

- Nulls. A metric that could not be computed is reported as such and excluded from the denominator, never coerced to zero.

- Instability. Any figure whose IQR exceeds 20% of its median carries an explicit unstable marker, per A.2.8.

- Precision. Timings to three significant figures; volumes to four; scores to two decimals, matching the precision the rule packs are authored at. No figure is reported to a precision its input does not support.

# A.4  Interpretation Guide

This section states, in advance, what the results will and will not license as a conclusion. It is written before the numbers exist so that the standard of proof cannot move to meet them.

## A.4.1  What the results mean for Blue Halo algorithm validation

Blue Halo generates a clearance envelope around each in-scope MEP element from a jurisdiction configuration, then reports elements intruding into it. Against synthetic fixtures it already passes 11 of 11 correctness checks, covering bounding-box expansion, intersection, clash detection, severity banding, angle-constraint enforcement, brace-variant handling, BCF structure and property-set serialisation. The 38-model run is not re-testing that arithmetic; it is testing four things the synthetic fixtures structurally cannot.

Table A.4.1 — What the run tests, and the criterion for each

| Claim under test | Evidence that supports it | Evidence that refutes it |
| --- | --- | --- |
| Geometry extraction survives third-party authoring | Halos generated for the large majority of MEP elements across models from eight repositories, two schemas and multiple authoring tools; halos_skipped attributable to identifiable causes (no placement, no shape representation) | Systematic skips concentrated in one authoring tool or one schema; envelope volumes implausible by orders of magnitude, indicating a unit-scaling fault |
| Unit handling is correct across projects | Envelope volumes on real models fall in a physically plausible band, consistent with the 13.5–21.5 m³ synthetic mean and explicable by element size where they exceed it | Volumes off by factors of 1,000 or 1,000,000 — the signature of a metre/millimetre scaling error surviving into production models |
| Cost scales linearly at production element counts | Halo generation time linear in element count; per-halo cost in the 234–490 µs band established at 100–20,000 elements | Super-linear growth, or per-halo cost rising with model complexity rather than with level of detail |
| Output is coordination-ready, not merely produced | BCF archives satisfying all seven conditions of Table A.2.7 and opening in a compliant reader; topic count equal to finding count on every model | Archives that open but whose topics reference component GUIDs absent from the source model — findings a coordinator cannot act on |

Two results are expected to be null, and are null by construction rather than by failure. The hospital-versus-standard clearance comparison will show no difference, because hospital_addition_mm is 0 mm in the configuration — a documented gap in the source standards, not a defect. Likewise, all four brace variants will produce identical envelopes, because neither EN 1998-1 nor DIN 4149 differentiates clearance by brace hardware. Reporting either null as a finding about the algorithm would be an error; reporting them as evidence that the standards data is incomplete is correct.

## A.4.2  What the results mean for corrosion engine effectiveness

The five engines are being assessed on three distinct properties, which must not be conflated. A high finding count is not by itself evidence of effectiveness — an engine that flags everything is as useless as one that flags nothing.

Table A.4.2 — Effectiveness dimensions

| Dimension | Question it answers | Measured by | What good looks like |
| --- | --- | --- | --- |
| Coverage | Can the engines obtain the inputs they need from real IFC files, as opposed to hand-built fixtures? | elements_scored ÷ element_count_mep, per engine, per material tier | High coverage on Full-tier models; low coverage on Partial-tier models accompanied by data-quality issues rather than silence |
| Discrimination | Do the engines separate benign from at-risk pairings, or do they return a flat result? | Band distribution per engine (Table A.3.1d); variance across the material heatmap (Figure A.2) | A spread of bands with an interpretable ordering; not all-Low and not all-Critical |
| Honesty | Does the system distinguish "assessed and benign" from "could not assess"? | data_quality_issues and unmapped_pairings counts | Unmapped pairings surface as explicit issues. Any unmapped pairing scored as compliant is a defect, not a pass |

The dominant expected finding is a coverage result rather than a risk result. Most models in the corpus carry geometry and system type but sparse material properties; only the Full-tier files (notably Clinic_Plumbing.ifc with 378 pipe entities carrying real material text for copper, PVC, CPVC, steel and brass, and the West Riverside plumbing and sprinkler models) can exercise the engines end to end. A result showing high risk-band output on Full-tier models and high data-quality-issue output elsewhere is the expected and correct behaviour of a system that refuses to default silently.

Three constraints bound what any corrosion result can claim. First, the scores are a calibration authored for these rulesets, not digits quoted from the cited standards, so results rank relative risk and do not certify engineering adequacy. Second, water chemistry and flow velocity are absent from the element model, so mechanisms dominated by them — copper Type 2 pitting in soft water, copper erosion-corrosion above roughly 1.5 m/s — are systematically understated. Third, the galvanic series underpinning GC-001 and XM-001 was verified against the payload as last committed, and one internally inconsistent adjacent pair (ss316_active versus copper) remains open. Any finding involving that couple is provisional.

## A.4.3  Expected outcome ranges

The ranges below are measured baselines from this codebase, not estimates. Blue Halo figures come from performance_benchmark.py at n = 7 repeats on a 4-core Linux host (Python 3.11.15, ifcopenshell 0.8.5, numpy 2.4.6, 15.7 GB RAM), reported as median with IQR. A real-model result inside the band is consistent with the baseline; outside it, the deviation is the finding and requires attribution to a stage.

Table A.4.3 — Measured baselines and expected ranges

| Metric | Synthetic / benchmark baseline | Expected range on real models | Out-of-range means |
| --- | --- | --- | --- |
| Parse time, single model | 1.52–1.55 s (IQR 0.09–0.20) at 100–1,000 elements | Scales with file size; 6.29 s observed for a 4-file federated set | Parse dominating total cost indicates a pathological STEP file, not an algorithm problem |
| Triangulation time | ≈ 30 ms per element (30.10 s for 1,000 elements) | The dominant stage on every real model | Expected. Triangulation is upstream of the claim under test and should be reported separately, never folded into halo cost |
| Halo generation throughput | 2,040–2,660 halos/s at LOD 300 from IFC; 3,910–4,274 halos/s synthetic | 2,000–4,000 halos/s | Below 1,000 halos/s: investigate geometry complexity or unit resolution per element |
| Per-halo cost | 376–403 µs at LOD 300; 19.9 µs at LOD 200; 490 µs at LOD 400 | 234–490 µs | Cost is driven by level of detail, not element count — a rise with element count indicates a super-linear term |
| Mean envelope volume | 13.47–21.47 m³ (49.45 m³ federated, larger elements) | Order 10–100 m³ | Off by 10³ or 10⁶: unit-scaling fault in the IFC path |
| Halo triangle amplification | 0.21×–0.63× source triangles at LOD 300–400; 0.02× at LOD 200 | 0.2×–0.7× | Above 1×: envelopes more complex than the geometry they wrap, indicating a generation fault |
| Broad-phase confirmation rate | 28–48% of candidate pairs confirmed by exact AABB test | 25–50% | Near 100%: the spatial hash is not filtering. Near 0%: the hash cell size is mistuned for the model |
| Spatial-hash speed-up vs naïve O(n²) | 0.5×–0.9× below 2,000 halos; 3.2×–12.9× from 2,000 to 20,000 | Speed-up only above ≈ 2,000 halos | Expected: on small models the hash costs more than it saves, and that is honest to report |
| Memory per halo | ≈ 2.1 kB (2.13 MB per 1,000 halos); RSS delta ≈ 0 below 20,000 halos | 1–5 kB per halo | Growth beyond this indicates retained geometry that should have been released |
| Blue Halo correctness checks | 11 of 11 passing on synthetic fixtures | Per-model equivalents all passing | Any failure is a defect, not a range excursion |
| BCF archive structure | 17 entries for 5 topics (3 files per topic plus bcf.version and project.bcfp); 9,905 bytes | 3n + 2 entries for n topics | Any other entry count fails condition 3 of Table A.2.7 |
| Corrosion coverage, Full-tier models | No real-model baseline exists; synthetic fixtures are 100% by construction | To be established by this run — the primary open question | This metric is the headline result, not a check |

The final row is deliberately open. Corrosion-engine coverage against third-party IFC files has no prior measurement in this project; establishing it is the principal contribution of the batch run, and pre-registering it as unknown prevents a post-hoc figure being presented as a confirmed expectation.

## A.4.4  Threats to validity

- Industrial category, n = 1. Only one model (craslabbim.ifc) is a genuine industrial or laboratory building. Any statement about industrial-facility behaviour is indicative and must be reported as such; the remaining five rows in that category are robustness testers, not a sample.

- Sample is convenience, not random. The corpus is what the open web makes freely available, which over-represents demonstration and benchmark models authored to be exported cleanly. Real project deliverables are typically messier, so parse and extraction success rates measured here are an optimistic bound.

- Vendor concentration. A large share of the corpus originates from Revit and ArchiCAD exports. Export-tool-specific conventions may therefore appear as general behaviour.

- No ground-truth clash set. Except for the published duct-length ground truth on wbdg_office (row 29), no model carries an authoritative list of true clashes. Precision and recall against a reference clash set cannot be computed; the run measures behaviour, agreement across schema twins, and internal consistency, not accuracy against a gold standard.

- Standards data is incomplete. Thirteen gaps are declared in the configuration metadata, seven of which change how results read (Table A.2.5b). Degenerate results traceable to those gaps are evidence about the standards extraction, not about the algorithm.

- Corrosion calibration is unverified by a domain expert. The rule-pack digits are authored, not quoted. A corrosion engineer must verify each cell against the cited source and against project water chemistry before any pack drives a compliance verdict.

- Single-host timing. Performance figures come from one 4-core host. They establish scaling behaviour and relative stage cost, not absolute performance on other hardware.

- Database drift risk. The galvanic series is loaded from a Supabase-hosted asset. Verification was performed against the payload as last committed because the live asset was unreachable; if the asset has since been edited, the engines read a different table than the one verified.

# A.5  Standards and Sources Cited in This Appendix

## A.5.1  Standards

ASCE 7-22 §12.10.4.2 — Minimum Design Loads and Associated Criteria for Buildings and Other Structures.

ASTM B117 — Standard Practice for Operating Salt Spray (Fog) Apparatus.

ASTM G48 Method B — Pitting and Crevice Corrosion Resistance of Stainless Steels and Related Alloys by Use of Ferric Chloride Solution.

ASTM G-187 — Standard Test Method for Measurement of Soil Resistivity Using the Two-Electrode Soil Box Method (MIC assessment context).

BS 8539 — Code of Practice for the Selection and Installation of Post-Installed Anchors in Concrete and Masonry.

BS 8552:2012 — Sampling and Monitoring of Water from Building Services Closed Systems.

buildingSMART BCF 2.1 — BIM Collaboration Format specification.

CIBSE Guide G — Public Health and Plumbing Engineering.

CIBSE TM13:2013 — Minimising the Risk of Legionellosis.

CIRIA C692 — Stainless Steel in Construction.

DIN 4149:2022-03 §8.2.4 — Buildings in German Earthquake Zones.

EN 1993-1-4 — Eurocode 3: Design of Steel Structures, Part 1-4: Stainless Steels.

EN 1998-1:2020 §5.3.2.3 — Eurocode 8: Design of Structures for Earthquake Resistance.

EN 12952-12 — Water-Tube Boilers: Requirements for Boiler Feedwater and Boiler Water Quality.

EN ISO 9308-1 — Water Quality: Enumeration of Escherichia coli and Coliform Bacteria.

EN ISO 15329:2007 — Crevice Corrosion Testing, Wetting Classes T0–T5.

HSE HSG274 Parts 1–3 — Legionnaires' Disease: Technical Guidance.

IMOA — Practical Guidelines for the Fabrication of Duplex Stainless Steel / Design Manual, 4th ed.

ISO 19650 — Organization and Digitization of Information about Buildings and Civil Engineering Works, Including BIM.

NASA-STD-6012 — Corrosion Prevention and Control Program Requirements.

NFPA 13-2021 §18.3.2.1 — Standard for the Installation of Sprinkler Systems.

WHO — Guidelines for Drinking-Water Quality, 4th ed.

## A.5.2  Dataset repositories

buildingSMART Community — Community-Sample-Test-Files. GitHub. CC BY 4.0.

Generalitat Valenciana, Conselleria d'Habitatge — BIM manual example models. Public-sector open data.

Hellin, S. — ifc-bench dataset. Hugging Face. CC BY 3.0. Mirrors the OpenIFC Model Repository (University of Auckland; models donated by Wawan Solihin) and the WBDG/NIBS Common BIM Files.

KIT — IFC Examples, via IFC Wiki. Free with attribution.

RWTH Aachen E3D — DigitalHub. GitHub. MIT Licence.

STEP Tools — Sample IFC Data Files (NIST/AISC CIS/2 conversions).

TIB Hannover — DuraArk BuildingData collection. Open data.

Zenodo record 7948116 — craslabbim.ifc, as-built laboratory model, FEUP. Open access.

## A.5.3  Project artefacts referenced

BIMGUARD AI Validation Dataset — Verified Downloadable IFC Models (repository root). Full inventory with verified download URLs.

hermes_standards_research_summary.json — extracted parameters for EN 1998-1, DIN 4149, NFPA 13 and ASCE 7-22.

hermes_case_study_and_config.json — combined EN 1998-1 + DIN 4149 clearance configuration, including metadata.data_gaps.

data/rulesets/mm_001_material_media.json — MM-001 compatibility matrix, approved v1.0.

data/rulesets/xm_001_cross_material.json — XM-001 cross-material pack, approved v1.0.

validate_blue_halo.py and validate_blue_halo_output_REAL_CONFIG.txt — synthetic validation harness and its 11/11 output.

test_real_ifc_pipeline.py — end-to-end IFC pipeline test on the Phase 5 fixture.

performance_benchmark.py and docs/benchmarks/halo_benchmark_summary.md — measured scaling baselines.

docs/defects/defect_report_anode_convention.md — resolution of the galvanic series sign-convention defect.